# 🔷 Logic Gate (A to Z)

Logic gates are the basic building blocks of digital electronics and computers.
They work using Binary Numbers:

- 0 = OFF / False

- 1 = ON / True

Logic gates take input signals and produce output according to logical rules.

# 🧠 Types of Logic Gates

There are mainly 7 basic logic gates:

- AND Gate

- OR Gate

- NOT Gate

- NAND Gate

- NOR Gate

- XOR Gate

- XNOR Gate

# 1️. AND Gate

### 📘 Definition

AND gate gives output 1 only when all inputs are 1.

### 🔹 Boolean Expression
Y = A . B

📋 Truth Table (Y=A . B)

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

### 🧾 Real Life Example

A machine starts only when:

- Power ON

- Switch ON

Both conditions must be true.

# 2️. OR Gate

### 📘 Definition

OR gate gives output 1 if any input is 1.

### 🔹 Boolean Expression
Y = A + B

📋 Truth Table (Y=A+B)

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

# 3️. NOT Gate

### 📘 Definition

NOT gate reverses the input.

### 🔹 Boolean Expression
Y = A̅

### 📋 Truth Table(Y = A̅)

| A | Y |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

# 4️. NAND Gate

### 📘 Definition

NAND = NOT + AND

It gives opposite result of AND gate.

### 🔹 Boolean Expression

### 📋 Truth Table

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

👉 NAND is called a Universal Gate because all gates can be made using NAND.

# 5️. NOR Gate

### 📘 Definition

NOR = NOT + OR

Opposite of OR gate.

### 🔹 Boolean Expression

### 📋Truth Table

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

👉 NOR is also a Universal Gate.

# 6️. XOR Gate

### 📘 Definition

XOR gives output 1 when inputs are different.

### 🔹 Boolean Expression

### 📋 Truth Table

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

# 7️. XNOR Gate

### 📘 Definition

XNOR gives output 1 when inputs are same.

### 📋 Truth Table

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

# 🔶 Universal Gates

Two gates are called universal:

- NAND Gate

- NOR Gate

# Because they can create all other gates.

what is Universal gate and why called them universal?
What is a Universal Gate?

A Universal Gate is a logic gate that can be used to create all other logic gates.

This means using only one type of gate, we can build:

- AND gate

- OR gate

- NOT gate

- XOR gate

- XNOR gate

- Any digital circuit

# ✅ Which Gates are Universal Gates?

There are two universal gates:

- NAND Gate

- NOR Gate

# 🔶 Why Are They Called “Universal”?

They are called universal because they are capable of performing any logical operation.

Using only:

- NAND gates → we can build all gates

- NOR gates → we can also build all gates

So, they are “universal” for digital circuit design.

# 🔷 K-Map (Karnaugh Map) A to Z

# 📘 What is K-Map?

K-Map (Karnaugh Map) is a graphical method used to simplify Boolean expressions.

It helps reduce:

- Logic gates

- Circuit complexity

- Hardware cost

# 🎯 Why K-Map is Used?

✅ Simplifies Boolean equations
✅ Reduces number of gates
✅ Makes circuit faster
✅ Saves cost and power

# 🧩 Types of K-Map

| Variables | Cells |
| --- | --- |
| 2 Variable | 4 Cells |
| 3 Variable | 8 Cells |
| 4 Variable | 16 Cells |

# 🔹 2 Variable K-Map

Variables: A, B

### Structure

| A\B | 0 | 1 |
| --- | --- | --- |
| 0 | m0 | m1 |
| 1 | m2 | m3 |

# 🔹 3 Variable K-Map

Variables: A, B, C

| A | BC=00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | m0 | m1 | m3 | m2 |
| 1 | m4 | m5 | m7 | m6 |

# 🔹 4 Variable K-Map

Variables: A, B, C, D

Contains 16 cells.

Used for larger Boolean simplification.

# 🧠 Important Terms in K-Map

## ✅ Minterm

A combination where output = 1

Example:
[
F(A,B)=\Sigma(1,2,3)
]

F(A,B)=\Sigma(1,2,3)

## ✅ Maxterm

A combination where output = 0

Example:
[
F(A,B)=\Pi(0,2)
]

F(A,B)=\Pi(0,2)

# 🔷 Rules of K-Map Grouping

Groups must contain:

- 1

- 2

- 4

- 8

- 16 cells

(Always power of 2)

# 📌 K-Map Simplification Example

## Example:
F(A,B)=\Sigma(1,2,3)

Truth table:

| A | B | F |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

After grouping:

Simplified equation:
F = A + B

F=A+B

# 🔶 Advantages of K-Map

✅ Easy simplification
✅ Saves hardware
✅ Reduces gate usage
✅ Faster digital circuit design

# 🔴 Limitations of K-Map

❌ Difficult for more than 5 variables
❌ Manual process
❌ Time-consuming for large circuits

# 📚 Final Revision Summary

## Logic Gates

- AND → All inputs true

- OR → Any input true

- NOT → Reverse

- NAND → Opposite AND

- NOR → Opposite OR

- XOR → Inputs different

- XNOR → Inputs same

## K-Map

- Used for Boolean simplification

- Uses grouping method

- Reduces circuit complexity

- Based on minterms and maxterms